AP010 Project 1 Spring 2011

For this project you must reserve and engineer your own session in Control A.  That includes session setup, mic and headphone setup.  Please make arrangements for musicians in advance.  All session guests must hold a current LAC ID.  Reserve your own session.  Do not work in groups.  Aim for a musical performance that you would be able to include on your demo reel.

Record a two to three minute musical performance of an acoustic instrument such as acoustic guitar, piano, violin, or trumpet.  Do not record electric guitar, electric bass, snare drum, percussion, or voice.  Simultaneously utilize close, distant, and ambient placement techniques.  Utilize spaced pair, XY, or M/S stereo technique on at least one of the placements.  Be sure to include a master stereo fader.  Do no apply any mixing or processing except proper panning and MS decoding if needed.  Try to set input levels that mix themselves.

Use 48kHz/24bit session parameters.

Session Organization:

Properly name the session.  For example J_Taylor_Project1

Properly label tracks what they are.  For example guitar, vocal, snare.

Use the comments view to detail mic types and placement.

Burn the entire session to CD or DVD-R properly labeled with a Sharpie marker – Your_Name_Project1, the date, the class, and the instructors name.

Sessions with missing files will receive a zero!

Due:  Thursday, May 19th.  Critiques:  Friday May 20th

Late Projects will not be accepted.