# AP301 Assignment 2_Field Mixer Anatomy

Part A:  Identify the front panel controls/indicators/features of a Sound Devices 302 Field Mixer

1. Fader - The primary control for adjusting gain during operation. Allows fine control over input levels

2. Peak LED - When illuminated, indicates that te channel is very near overload.

3. Limiter LED - When illuminated, indicates that the channel limiter is operating and reducing channel gain to prevent overload.

4. Gain(Trim) - The coarse gain control. Sets the input range so that fine gain control can be applied with the channel fader.

5. Polarity Reverse switch - When engaged, the polarity of channel 2 is reverse (180 degrees out of phase) with respect to input channels 1 and 3.

6. limiter Switch-Activates both input and output limiters. ON is dual-mono limiter operation, LINK is stereo operation. Output limiter threshold controlled in user Setup menu.

7. Output Meter - Sunlight-viewable, 40-segment LED output meter indicates peak, VU, or combination peak/VU output levels. Calibrated in dBu when peak-reading.

8. Mic/Tone Switch - Two-position switch activates either the slate mic in the left-momentary position, or a tone oscillator in the right-latched position. Additional options available in the user setup menu.

9. Pan Switch - Assigns the input channel to the output bus. Left-only, Center (equal left and right), or Right-only.

10. High-Pass Filter-Three-Position switch engages the high-pass shelving filter. Used to reduce excessive low frequencies. 12 dB per octave at 80 Hz or 160 Hz.

11. Stereo Link LED - Indicates that channels 1 and 2 are linked as a stereo pair. Engaged via a user setup. In L/R stereo link channel 2 fader controls overall stereo level. When in MS position channel 1 fader controls Mid, channel 2 fader controls the amount of stereo (side) information.

12. Meter Brightness - Controls the brightness of the LED output meter. Each push selects among four brightness levels

13. Meter Ballistics - Toggles among the available meter ballistics options: VU-only, peak-only, peak-hold/VU.

14. Headphone Selector - Sets the signal source sent to headphones. Options include: PFL 1, 2, 3; left output bus; right output bus; Mono (summed left and right); Stereo master; RTN-stereo monitor return; MS-Mono; MS-stereo; RTN-MS

15. Headphone volume - Adjust the overall volume of the headphones

16. Headphone LED - Indicates overload of the headphone circuit and RTN circuit.

17. Battery Check Button – Depressing indicates both internal and external battery levels on the output meter. Battery level is shown for two seconds after button pressing

18. Power Switch/LED – Three-position switch selects between internal battery power or external DC sources. Power LED illuminates when power is on. LED flashes when voltage reaches low limit.

Part B:  Identify the input panel controls/features/indicators of a Sound Devices 302 Field Mixer

1. Transformer-Balanced Inputs – Can be used unbalanced by grounding pin-3 to pin-1 of the XLR connector. Pin-1 = ground; pin-2 = hot; pin-3 = cold.

2. Mic/Line Channel Switch – Selects the input of the adjacent connector. Mic level has 40 dB more gain than line level.

3. Phantom/Dynamic/T-Power selection – Selects the microphone powering type of the adjustment input. DYN position turns off all microphone powering. Mic powering is selected per input.

4. Phantom Voltage Selection – Selects between 48 V or 12 V phantom voltage for the input channels. The three-position switch uses two positions for 12V-there is no difference between these positions.

5. Headphone Output – 1/8-inch TRS stereo headphone output. Can drive headphones from 8 to 2000 ohms to required monitoring levels.

Part C:  Identify the output panel controls/features/indicators of a Sound Devices 302 Field Mixer

1. XLR Master outputs – Active-Balanced Outputs. Pin-1 = ground; pin-2 = hot; pin-3 = cold. Can be unbalanced by using pin-2 for signal and pin-1 for ground.

2. Battery Tube – Holds three-AA batteries for internal powering.

3. Return level (Channel 4/5 level) Control – Adjusts the gain of the return feed to balance program and monitor signals in headphones.

4. Return (Channels 4/5) input – Unbalanced stereo 1/8-inch input connector for return monitor audio. 1/8-inch wired tip = left, ring = right, sleeve = ground. Connection used for inputs 4 and 5 when selected in Setup menu.

5. Mix-in – An input to the master bus designed exclusively to link the Tape Out/Mix Out of 302, 442, Mix Pre, or MP-2 to the 302 for additional inputs. Pin-1 = ground, pin-2 = left, pin-3 = right. Shell of a TA3 connector must be grounded to pin-1 to open connection.

6. Channel 4/5 Activation LED – When illuminated, indicates that the return connector is now set as an input 4 and 5 connections.

7. Output Attenuation LED – When illuminated, indicated that the XLR output connectors are set for a level other than the factory default of line level.

8. Tape Out / Mix Out – Unbalanced stereo output on TA3-type connector. Same program as master output. Pin-1 = ground, pin-2 = left, pin-3 = right. Also used to link to the Mix in of a 442 or 302.

9. DC input – Accepts DC voltage from 5 -18 V for mixer powering. Four-pin Hirose connector wired pin-4 positive(+), pin-1 negative (-). Ext DC is completely isolated (floating) from the rest of the circuitry.